# Lab 4 — Instructor Key: Bending

For instructors only. Do not distribute to students. Every number in this key is computed by src/lab04_analysis.py (and src/make_key.py) from sample_data.csv and src/specimen_measurements.json, or by the simulation’s own model code run in Node. To build a key for your own class data, put your CSV and measurements in the same format and run python3 src/lab04_analysis.py your.csv your_measurements.json, then python3 src/make_key.py. Uncertainties are expanded (U=2u) unless marked u. Sign convention as in the handout (Beer 8e): V and M as in Fig. 5.6, M>0 sagging, σx=−My/I with y up from the neutral axis, tension positive.

## 1. Results at a glance (sample data)

Table K1. Results from the sample data.

| Quantity | Measured | Reference or prediction | Verdict (Eq. 18) |
| --- | --- | --- | --- |
| Bar A: b, h (mm); I (105 mm4) | 25.420 ± 0.026, 50.780 ± 0.026; 2.7738 (uI = 0.092 %) | 25.4 × 50.8 nominal | — |
| Shear span a (mm) | 200.00 ± 0.41 | 200 | — |
| V/P in the left shear span (A1) | 0.5005 ± 0.0242 | 0.5 (Eq. 2) | agree |
| Neutral axis, bar A (mm above bottom) | 25.392 ± 0.026 | h/2 = 25.390 | agree |
| E of 6061-T6 from M–κ (GPa) | 68.39 ± 0.90 | Lab 2: 68.46 ± 0.94 | agree |
| ν from anticlastic strain | 0.3294 ± 0.0059 | Lab 2: 0.330 ± 0.006 | agree |
| Neutral axis, bar B (mm above bottom) | 23.985 ± 0.039 | Eq. 9: 23.95 ± 0.18 | agree |
| EI, bar B (N·m²) | 38 606 ± 64 | E1It: 38 751 ± 808 | agree |
| M at 40 kN and κ = 0.2935 m−1 (N·m) | 4 000 | EPP 4 054; material curve 4 003 | EPP +1.34 %; curve +0.07 % |
| Flexure stress vs actual at 40 kN (MPa) | actual 282.0 | Mc/I = 366.2 | overstated 29.8 % |
| Permanent curvature (m−1) | 0.0827 | Eq. 14: 0.0826 ± 0.0028 | agree |
| Straightedge gap after Part C (mm) | 2.85 ± 0.05 | middle 200 mm only: 2.48 ± 0.09; whole bar from the measured M–κ: 2.87 | gap 15 % larger: shear spans yielded too |
| D1 lightest acceptable section | steel I-120, aluminum I-160 | — | — |

The sample data were generated from a known “truth” model (src/generate_sample_data.py, values in src/truth.json): plane sections, 6061-T6 following Lab 1’s Ramberg–Osgood curve (E = 68.9 GPa), the steel strip elastic, a common +0.5 % gage-factor error, true gage heights that differ from the recorded ones by a few hundredths of a millimeter, load points 0.4 mm inside their nominal positions, and 1 µε reading noise. Because every strain reads 0.5 % high, the measured E is 0.5 % low: correct for the gages, invisible to the student, and inside the stated uncertainty. The closure test in tests/test_lab04_independent.py confirms that the analysis recovers the truth values within their stated uncertainty.

## 2. Pre-lab solutions

P1. (a) I=bh3/12 = 2.7749 × 105 mm4, S = 10 925 mm³. At 20 kN, M=P/2a = 2 000 N·m, σm=M/S = 183.1 MPa, ε=σ/E = 2 674 µε, κ=M/EI = 0.1053 m−1, ρ = 9.50 m. (b) MY=σYS = 2 990 N·m, so PY=2MY/a = 29.90 kN; Mp=1.5MY gives 44.85 kN. Part C’s 40 kN is 1.338 PY and 0.892 Pp: well into the plastic range but short of collapse. (c) At 2.5 kN the bottom strain is about 334 µε, so 1 µε is 0.30 %. (d) V/bh = 7.75 MPa, 4.2 % of the bending stress (the parabolic maximum, 11.6 MPa, is Lab 5’s subject). Units: N·m/mm³ → MPa after ×10³; µε = 10−6.

P2. (a) Figure 2: V=P/2, 0, −P/2; integrating, M rises to Pa/2, stays flat, and falls by Pa/2 to zero at L. (b) a1+a2=L, so the loads are still symmetric: RA=RB=P/2, and M=P/2a1 = 100.200 × 10−3 P (m), 0.200 % above Eq. 2. The middle is still in pure bending. (c) RA = 0.50033 P and RB = 0.49967 P (Appendix D.1); between the loads V=RA−P/2 = 0.000333 P, so M changes by 0.067 % across the middle third. Strictly not pure bending, but far below anything the gages can see. Lesson: the measurement is insensitive to small load-point errors because the moment in the middle depends on a only to first order.

P3. n=205/68.46 = 2.9944. With y from the bottom of the strip, transformed areas nbts at ts/2 and bh at ts+h/2: y = 23.967 mm and It = 5.6607 × 105 mm4 (bar alone: 2.7749 × 105 mm4; ratio 2.040). At 15 kN, M = 1 500 N·m: steel bottom +190.2 MPa; interface aluminum +46.68 MPa and steel +139.8 MPa (ratio n); aluminum top −87.9 MPa. (b) ts→0: y→h/2 and It→bh3/12. n=1: a homogeneous bar of depth h+ts, with y=h+ts/2.

P4. (a) MY = 2 990 N·m, κY=σY/Ec = 0.1574 m−1, Mp = 4 485 N·m. (b) M/MY = 1.3378; Eq. 12 gives κY/κ2=31−M/1.5MY, so κY/κ=yY/c = 0.5696, κ = 0.2763 m−1, and the elastic core is ±14.47 mm. (c) Elastic recovery M/EI = 0.2106 m−1, so κp = 0.0657 m−1. Residual stress: top −σY+Mc/I = −273.7 + 366.1 = +92.4 MPa (tension), bottom −92.4 MPa. (d) −∫yσres dA=M−M=0 (Appendix D.7); students may also integrate the piecewise-linear residual distribution numerically.

P5. Simulation, presets as shipped (nominal section, E = 68.9 GPa). Part A: M = 2 000 N·m, largest σ = 183.1 MPa. Part B: elastic NA 24.004 mm, It = 5.6472 × 105 mm4 (slightly different from P3 because the page uses E = 68.9 GPa). Part C at κ = 0.292 m−1: material curve 4 008 N·m, EPP 4 060 N·m (+1.28 %). The EPP law is larger: aluminum’s curve bends over before σY (the 0.2 % offset point), so the outer fibers carry less than σY just past yield, and Part C’s strains are not yet large enough for the hardening to make it up. “Overload, then unload” (κ=3κY = 0.4697 m−1, M = 1.4444 MY): permanent curvature 0.2435 m−1.

P6. (a) Pinch points at the rollers and spreader (hands clear when the crosshead can move); stored elastic energy if the bar shifts or a roller slips (bar centered and square, stop on a load drop); overloading the bar or fixture (overload stop set per part); manual handling of 2.5 kg bars. (b) Part C bends the bar permanently, so it must come after Part A, which needs an undamaged bar; an overload stop set before the plastic part is the only protection against running past 40 kN toward collapse at about 44.85 kN, where the deflection would grow quickly.

## 3. Analysis solutions

A1. Slopes of bottom strain against P converted with M=ESε (E from A3): M/P = 49.91 mm at x = 100 mm (statics 50.0, −0.18 %), 74.94 mm at 150 mm (75.0, −0.08 %) and 100.06 mm at midspan (100.0, 0.06 %); each ±0.67 % (u, mostly from E). V/P = 0.5005 ± 0.0242, Δ = 0.0005 ≤ 0.0242: agree. Check: the line through the two shear-span points meets M=0 at x = 0.28 mm, at the support, as a pin must require. Interpret: the slope of the moment diagram is the reaction (V=RA in the shear span); between the loads both load points’ halves cancel, so the slope, V, is zero. The shear is a difference of two nearly equal moments, which is why its relative uncertainty (about 2.4 %, u) is several times that of M.

A2. 11 steps above 1 kN. Curvature: 0.02637 m−1 at 5 kN, 0.05268 at 10 kN, 0.10540 at 20 kN (linear in load). RMS residuals 0.79–3.10 µε (1.81 at 20 kN). At 20 kN, top −2 675 µε and bottom 2 677 µε. yNA = 25.392 ± 0.026 mm vs h/2 = 25.390 mm: difference 0.002 mm, within 0.026. Check: residuals at the 1–3 µε level with no curvature in the profile; equal and opposite extreme strains. Interpret: a curved profile would mean sections warp (or a gage is misplaced); residuals growing with load in the same pattern would mean gage-height errors (C3), not physics.

A3. 11 loading steps; slope EI = 18 971 N·m². E = 68.39 ± 0.90 GPa vs Lab 2 68.46 ± 0.94: Δ = 0.07 ≤ 1.30, agree. Budget (u, %): fit 0.031, b 0.051, 3uh/h 0.076, a 0.102, load cell 0.289, gage factor 0.577; total 0.661 %. At 20 kN: flexure formula 183.0 MPa vs Eε = 183.1 MPa (ε = 2 677 µε). Check: slope units N·m/(m−1) = N·m²; I∝h3 so 1 % in h is 3 % in E. Interpret: the gage factor dominates. A common gage-factor error scales every strain, so it scales κ and biases E, but it cancels in yNA=−A0/A1 (C3).

A4. ν = 0.3294 ± 0.0059 vs 0.330 ± 0.006: agree. At 20 kN ρ = 9.49 m, so ρ′=ρ/ν = 28.8 m. Check: the compressed top widens, so the transverse gage there reads positive (882 µε at 20 kN). Interpret: the anticlastic curvature makes the top face slightly saddle-shaped; near the edges the transverse strain is affected by the free edges and the gage’s finite length, so a centered gage is best (§4.3 assumes the section is free to deform).

A5. Measured NA 23.985 ± 0.039 mm vs Eq. 9 with measured dimensions and n = 2.9974: 23.95 ± 0.18 mm; Δ = 0.033 ≤ 0.186, agree (the prediction’s uncertainty is dominated by Esteel). All-aluminum bar of the same depth: 28.57 mm. EI = 38 606 ± 64 N·m² vs E1It = 38 751 ± 808 (−0.38 %), agree. At 14.993 kN (M = 1 499.3 N·m), from the fitted strain line (RMS 1.36 µε): steel bottom 190.8 MPa (predicted 190.0, 0.41 %), aluminum interface 46.8 (46.6), steel interface 140.1 (139.5), aluminum top −88.1 (−87.8). ∫σ dA = 0.17 % of the compressive resultant. Check: it must vanish in pure bending (first of Eqs. 4); the small remainder is the strain-line fit and the 0.5 % gage-factor bias, which cancels in ratios. Interpret: the composite is 2.035 times as stiff as bar A (18 971 N·m²); the steel’s bottom fiber is the most highly stressed point, about twice the aluminum’s top.

A6. Table K2 lists the Part C steps. MY = 2 990 N·m, Mp = 4 485 N·m, κY = 0.1576 m−1 (measured E, Lab 1 σY).

Table K2. Part C (loading branch and unloading); model moments at the measured curvature.

| P (kN) | M (N·m) | κ (m−1) | EPP, Eq. 12 (N·m) | Material curve, strips (N·m) | yNA (mm) | RMS (µε) |
| --- | --- | --- | --- | --- | --- | --- |
| 5.0 | 501 | 0.0264 | 500 | 500 | 25.40 | 0.3 |
| 10.0 | 1 000 | 0.0526 | 999 | 999 | 25.40 | 2.0 |
| 15.0 | 1 501 | 0.0791 | 1 500 | 1 500 | 25.40 | 2.2 |
| 20.0 | 2 000 | 0.1054 | 1 999 | 1 999 | 25.40 | 2.3 |
| 25.0 | 2 501 | 0.1323 | 2 509 | 2 499 | 25.39 | 3.9 |
| 28.0 | 2 801 | 0.1500 | 2 846 | 2 800 | 25.39 | 4.1 |
| 31.0 | 3 100 | 0.1712 | 3 218 | 3 100 | 25.39 | 4.5 |
| 34.0 | 3 400 | 0.1986 | 3 544 | 3 401 | 25.39 | 4.8 |
| 37.0 | 3 700 | 0.2365 | 3 821 | 3 702 | 25.39 | 5.7 |
| 40.0 | 4 000 | 0.2935 | 4 054 | 4 003 | 25.39 | 6.6 |

At 40 kN (M/MY = 1.338): measured κ = 0.2935 m−1, extreme strain 0.745 %. EPP predicts 4 054 N·m (+1.34 %), the strip summation with Lab 1’s curve 4 003 N·m (+0.07 %). Flexure stress 366.2 MPa vs actual 282.0 MPa: 29.8 % too high. Permanent curvature: measured 0.0827 m−1, Eq. 14 0.0826 ± 0.0028: agree (elastic recovery 0.2109 m−1). The zero-load surface strains, ±2 099 µε, are κpc. Straightedge gap: measured 2.85 ± 0.05 mm; Appendix D.9 with κp confined to the middle 200 mm gives 2.48 ± 0.09 mm. The difference, 0.37 mm (limit 0.10), is real: the moment exceeds MY in the shear spans beyond x=MYa/Mmax = 149.5 mm, so the bar also yielded near the load points. Integrating κpx=κMx−Mx/EI along the whole bar, with κM interpolated from the measured loading curve, gives 2.87 mm, in agreement. (Expect only the first estimate from most students; the second is worth credit as an interpretation.) Surface residual stress on the tension side (strip summation, material curve): −84.2 MPa. Check: yNA = 25.39 mm at 40 kN and the RMS residual 6.58 µε vs 1.81 at 20 kN in Part A. Per unit curvature, 22.4 vs 17.2 µε·m: the residual grows with κ in the same pattern, the signature of gage-height errors (C3), so plane sections still hold. Interpret: aluminum’s curve bends over before the 0.2 % offset point, so near yield the outer fibers carry less than σY and EPP overpredicts M. The same gradual yielding means the measured curvature at 4000 N·m is larger than EPP’s 0.2768 m−1, so the permanent set (0.0827) is 1.25 times the bilinear EPP estimate (0.0659 m−1). Also worth credit: the transverse-to-longitudinal strain ratio on the top face rose to 0.408 at 40 kN (plastic flow is incompressible, so the ratio heads toward 0.5); the bottom gage at x = 150 mm, where M≈MY at 40 kN, kept 138 µε of 4 144 µε: the plastic zone reached just to it, consistent with the gap.

A7. Use Table K1. Expected verdicts: every Table 5 row agrees except two model-form discrepancies. The moment at 40 kN: EPP is +1.34 % (the comparison’s U is about 1 %), and the material curve removes it. The straightedge gap: the middle-only estimate of D.9 is 15 % low because the shear spans yielded too (A6). Full credit needs each discrepancy above U explained with a named, quantified cause.

M1. (a) Flexure formula vs actual extreme-fiber stress (simulation, material curve): 30 kN: 275 vs 252 MPa (9 %); 35 kN: 320 vs 268 MPa (19 %); 40 kN: 366 vs 282 MPa (30 %). At the preset curvature: 366.9 vs 282.3 MPa, 30.0 % high (the readout’s Difference column shows 23 % because it divides by the flexure value; accept either if the base is stated). The error grows quickly beyond PY because M keeps rising while the outer-fiber stress barely does. (b) At 40 kN’s curvature, EPP is 1.34 % above the material curve; the data follow the material curve (0.07 %). (c) With the strip made of aluminum the neutral axis moves from 24.00 to 28.58 mm (+4.57 mm), and at M = 1500 N·m the bottom stress would be predicted as 108.5 MPa instead of 189.7 MPa: ignoring the strip underestimates the steel’s stress by a factor of 1.75. (d) The RMS residual scales with κ (A6), so compatibility (Eq. 3) holds through Part C. What failed is Hooke’s law: Eqs. 5–6 assume a linear σ–ε relation, and Eq. 12 assumes a flat plateau.

## 4. Challenge and Extension solutions

C1. Appendix D.6: Z=btfh−tf+twhw2/4 = 238.6 × 10³ mm³, S = 209.8 × 10³ mm³, k = 1.1371 (rectangle 1.5). The experiment (κ = 16.0 κY) gives M/MY = 1.1368, essentially fully plastic. MY = 52.5 kN·m, Mp = 59.7 kN·m. In an I-section most of the material is already near ±σY at first yield, so there is little left to mobilize; a rectangle’s lightly stressed core can still pick up a lot. Design against collapse (plastic design) therefore gains 50 % for a rectangle but only about 14 % for an I-section; the I-section is still far more efficient per unit mass.

C2. Part B (elastic): errors Ns = 10: −8.17 × 10−3, Ns = 20: −2.05 × 10−3, Ns = 40: −5.17 × 10−4; successive ratios 3.985 and 3.964 (4 for second order). Richardson from 20 and 40: relative error −6.21 × 10−6. Part C with EPP: Ns = 10: −3.39 × 10−3, Ns = 20: −3.36 × 10−4, Ns = 40: −2.58 × 10−4, ratios 10.1 and 1.3; the least-squares slope over Ns = 2–256 on the plot is 2.09. The midpoint rule is second order for a smooth integrand; the EPP stress has a kink at ±yY, and the error contributed by the strip containing the kink depends on where in that strip the kink falls, so the error oscillates about an Ns−2 trend. Triangular load: N = 2: 6.25 × 10−2, N = 4: 1.56 × 10−2, N = 8: 3.91 × 10−3, N = 16: 9.77 × 10−4, N = 32: 2.44 × 10−4 (slope 2.00). Four-point bending: the shear is piecewise constant and the load points are nodes, so the trapezoid rule integrates it exactly at any N (largest error 0.0).

C3. (a) Every strain scales by 1+δ, so A0 and A1 both scale and yNA=−A0/A1 is unchanged while κ scales. (b) Replacing yh by yh+d changes A0 by −A1d and leaves A1 unchanged. (c) For one gage, δA1/A1=−δyjyj−y/∑yi−y2 to first order; with ∑yi−y2 = 2 288 mm², a 0.1 mm error changes κ by at most 0.089 %. At 20 kN it is equivalent to a 10.5 µε strain error at that gage, which is how A2’s residuals (1–3 µε) bound the gage-height errors to a few hundredths of a millimeter. None of these explains a discrepancy here; the gage factor (a) is the largest term in A3.

X1. Strains ε=ε0−κyh in both layers (compatibility, perfect bond); stresses Eε−αΔT; N=0 and M=0 give two linear equations for ε0 and κ. With measured dimensions and E: κ/ΔT = −0.2114 × 10−3 m−1/K, hogging (concave down) because the aluminum on top expands more. At 50 K: κ = −0.0106 m−1 (radius 94.6 m); interface stresses steel +50.6 MPa and aluminum −23.8 MPa; steel bottom 36.8 MPa; aluminum top 12.9 MPa. Check: equal coefficients give κ=0 and no stress. The interface shear needed to transfer these forces is concentrated near the ends, which is where bimetal bonds fail.

X2. Unloading from Mmax leaves σresc=−σY+Mmaxc/I=σYMmax/MY−1 on the compression side (tension), and the opposite on the tension side. On reverse loading by ΔM, the extreme fiber reaches −σY again (on the original tension side) when σY1−Mmax/MY−ΔMc/I=−σY, i.e. after a total change of 2MY from Mmax: reverse yield at M=Mmax−2MY. Since Mmax≤Mp=1.5MY<2MY, unloading to zero never reverse-yields (the residual is at most 0.5 σY). Reloading in the original direction is elastic up to Mmax (shakedown; the same idea as the autofrettage of gun barrels and the presetting of springs).

X3. T-section from the I-beam preset (flange 100 × 10 mm, web 6 mm, depth 200 mm, area 2 140 mm²): elastic NA 141.73 mm above the bottom; plastic NA 178.33 mm (equal areas, in the web just below the flange); strip summation at 104κY puts it at 178.33 mm with M/Mp = 1.00000. Z = 112.5 × 10³ mm³, S (to the extreme fiber, at the bottom of the web) = 61.8 × 10³ mm³, k = 1.819. The elastic axis balances first moments of area; the plastic axis balances areas. They coincide only for a section symmetric about the neutral axis.

## 5. Design problem D1: reference solution

D1. Mmax=wL2/8+PL/4 = 7.125 kN·m at midspan; δallow = L/240 = 12.5 mm. Steel: σY−U = 294.0 MPa, allowable 176.0 MPa, Sreq = 40.47 × 10³ mm³, Ireq = 2.332 × 106 mm4 (E = 205 GPa). Aluminum: 271.7 MPa, 162.7 MPa, Sreq = 43.79 × 10³ mm³, Ireq = 6.991 × 106 mm4 (E = 68.39 GPa, measured).

Table K3. Steel candidates (stress is the same for both materials).

| Section | Mass (kg/m) | σmax (MPa) | δ, steel (mm) | Acceptable |
| --- | --- | --- | --- | --- |
| I-100 | 7.79 | 218.1 | 17.85 | no |
| I-120 | 10.07 | 139.6 | 9.52 | yes |
| I-140 | 12.60 | 94.9 | 5.55 | yes |
| I-160 | 15.26 | 68.3 | 3.49 | yes |
| I-180 | 18.30 | 50.4 | 2.29 | yes |
| I-200 | 21.44 | 38.6 | 1.58 | yes |
| RT-100x50x4 | 8.94 | 247.2 | 20.23 | no |
| RT-120x60x5 | 13.38 | 138.2 | 9.42 | yes |
| RT-150x100x5 | 18.89 | 70.8 | 3.86 | yes |
| RT-200x100x6 | 27.20 | 39.7 | 1.63 | yes |

Table K4. Aluminum candidates.

| Section | Mass (kg/m) | σmax (MPa) | δ, aluminum (mm) | Acceptable |
| --- | --- | --- | --- | --- |
| I-100 | 2.67 | 218.1 | 53.50 | no |
| I-120 | 3.45 | 139.6 | 28.53 | no |
| I-140 | 4.32 | 94.9 | 16.64 | no |
| I-160 | 5.24 | 68.3 | 10.47 | yes |
| I-180 | 6.28 | 50.4 | 6.87 | yes |
| I-200 | 7.36 | 38.6 | 4.73 | yes |
| RT-100x50x4 | 3.07 | 247.2 | 60.63 | no |
| RT-120x60x5 | 4.59 | 138.2 | 28.24 | no |
| RT-150x100x5 | 6.48 | 70.8 | 11.58 | yes |
| RT-200x100x6 | 9.33 | 39.7 | 4.87 | yes |

Steel: I-120, 10.07 kg/m (30.2 kg, about $66); utilization 0.79 in stress and 0.76 in deflection, so the two requirements are nearly balanced. Aluminum: I-160, 5.24 kg/m (15.7 kg, about $102); utilization 0.42 in stress and 0.84 in deflection: deflection governs, because aluminum’s E is a third of steel’s while its strength is similar. The aluminum beam is about half the mass at about 1.5 times the cost; either is acceptable if justified. Assumptions that could invalidate a design: lateral–torsional buckling of the unbraced compression flange over 3 m (check bracing, or a buckling check that comes later in the course); local buckling of thin flanges (the catalog proportions are compact); load placement other than at midspan; long-term creep is not an issue for these metals at room temperature.

Rubric detail (15 points total in the report). Moment and requirements (3). Strength basis (3.75): σY−U from the student’s Lab 1 data and measured E for aluminum. Every requirement checked, governing one named for each material (3). Trade-off between at least two options with numbers (3.75). Presentation and assumption (1.5). Typical partial credit: w in kN/m mixed with mm (−2); deflection not checked (−4, it governs aluminum); handbook values instead of measured (−1); S and I swapped (−2).

## 6. Expected ranges for real class data

Table K5. Expected data ranges.

| Quantity | Expected | If outside, suspect |
| --- | --- | --- |
| RMS residual of the profile fit, Part A | 1–5 µε | a gage misplaced or debonded; wrong height recorded |
| yNA, bar A | h/2 ± 0.3 mm | gage heights measured from the wrong face; bar not seated (twist) |
| E of 6061-T6 (GPa) | 67–71 | gage factor setting; h measured at the wrong place; a wrong |
| ν from the top-transverse gage | 0.31–0.35 | gage not centered or not transverse |
| V/P from the bottom gages | 0.45–0.55 | gage x positions; local effect of the load roller at x = 150 mm |
| Bar B NA (mm above bottom) | 23.5–24.5 | adhesive layer thickness; strip not 1018 |
| Permanent curvature after Part C (m−1) | 0.06–0.10 | plateau of the particular bar; load overshoot |

## 7. Common student mistakes

- Measuring gage heights from the top instead of the bottom, or mixing y (from the neutral axis) with yh (from the bottom).

- Using σ=Mc/I with c measured to the far fiber of the composite instead of using the transformed section and the factor n.

- Forgetting that κ=−A1: a positive slope of strain against height means hogging.

- Fitting M against κ through the unloading steps or the zero step, or forcing the line through the origin without saying so.

- Using P instead of P/2 in M=P/2a.

- Computing E from a single bottom gage with the handbook neutral axis, which hides the plane-sections test.

- Applying Eq. 6 in Part C and calling 366 MPa the stress in an aluminum bar whose tensile strength is about 314 MPa.

- Taking the permanent curvature as κmax−κY.

- In A5, comparing the steel stress with −My/It without the factor n.

- In D1, checking stress only, or using the steel modulus for the aluminum beam.

## 8. Grading rubric

Table K6. Grading rubric (100 points plus up to 10 Challenge points). Core analysis A1–A7 totals 45.

| Part | Points | Full credit | Common deductions |
| --- | --- | --- | --- |
| Pre-lab | 15 | P1–P6 correct with units; P1 an estimate made before measuring | P2(c) missing (−1); P4 inversion wrong (−2) |
| Data & procedure | 10 | Tables 3–4 complete; checks in steps 1–6 recorded | gage heights not measured (−3) |
| A1 | 6 | Mx from gages vs statics, V/P with U, intercept check | P/2 error (−2) |
| A2 | 7 | profiles, κ, yNA with U, residual discussion | no residuals (−3) |
| A3 | 7 | E with budget, comparison with Lab 2, flexure check | no budget (−3) |
| A4 | 5 | ν with U, ρ′, sign check | — |
| A5 | 8 | NA, EI, four stresses vs prediction, ∫σ dA check | factor n missing (−3) |
| A6 | 8 | M–κ with both models, flexure overstatement, permanent curvature, residual stress | elastic formula past yield (−3) |
| A7 | 4 | complete table, normalized figure, every discrepancy explained | unexplained discrepancy (−1 each) |
| M1 | 10 | (a)–(d) with simulation numbers and the plane-sections argument | no simulation numbers (−2 each) |
| D1 | 15 | see Sec. 5 | see Sec. 5 |
| Communication | 5 | labeled axes, units, x±U format | each unlabeled axis (−1) |
| Challenge | +10 | C1 (3), C2 (4), C3 (3) | — |

Every Core answer must include the required check and interpretation; deduct 1 point per question where either is missing.

## 9. Instructor notes

Before the lab. Bars: 6061-T6, 25.4 × 50.8 mm (1 × 2 in) bar stock, 700 mm long. Bar A gets ten gages (five on one side face at 5, 15, 25.4, 35 and 45 mm, top, bottom and top-transverse at midspan, and two bottom gages at 100 and 150 mm from the left support); bar B gets eight. Bond the 6.35 mm 1018 strip to bar B with a structural epoxy (thin, uniform bond line; clamp while curing). Part C consumes one bar A per section run: either give each team its own bar (about 2.5 kg of 6061 bar per team) or run Part C once as a demonstration and share the data. Check the fixture’s rollers turn freely; seized rollers add axial force and spoil the neutral axis.

Timing (2 h). 0:00–0:10 safety and pre-lab collection; 0:10–0:25 dimensions and gage heights; 0:25–0:55 Part A (eleven steps with holds); 0:55–1:20 Part B; 1:20–1:45 Part C; 1:45–2:00 straightedge check and simulation (step 8).

Safety. Demonstrate the overload stop and a controlled stop. Under 40 kN the 6061 bar deflects only a few millimeters, but a spreader that tips releases the load suddenly; insist on the centering check.

Running without equipment. Give students sample_data.csv and Appendix A of the handout; everything in this key applies unchanged. A good hybrid is to run Part A live (it is non-destructive) and use the sample data for Parts B and C.

Your own class data. Export the columns listed in Table A1 of the handout, one row per load step; fill a measurements JSON like src/specimen_measurements.json, with the class’s Lab 1 and Lab 2 values in reference; run src/lab04_analysis.py and src/make_key.py.

The simulation. Students use it in P5, step 8, A6, M1, C1, C2 and X3. Its readout separates verification rows (closed form vs. integration or strip summation, “agree”) from model-comparison rows (“different models”); the tests are in tests/ and summarized in tests/VERIFICATION.md.

## 10. Self-critique and revisions

Before release, the lab was reviewed as a demanding instructor at a top program would review it. The three biggest weaknesses found, and the fixes made:

- The planned lab verified the flexure formula with a handbook modulus and an assumed neutral axis, which is circular: it can only “confirm” what it assumes. Fix: seven gages through the depth measure the strain profile, so the neutral axis and curvature come from the data without assuming the material is elastic (Eq. 15). E is measured from moment and curvature and compared with Lab 2, a transverse gage measures ν (A4), and the residuals test plane sections directly (A2, M1(d)).

- Shear and moment diagrams (§5.1–5.2) were drawn but never measured. Fix: two bottom gages in the shear span turn strain into Mx and the moment gradient into V (Eq. 16, A1), with an intercept check at the support. P2 adds the sensitivity to load-point placement.

- No test of where the formulas fail. Fix: Part B tests the transformed section on a real two-material beam, including the equilibrium check ∫σ dA=0 from measured strains (A5). Part C loads past first yield and compares the measured moment–curvature curve with the EPP formula and with the material’s own curve, quantifies the 29.8 % overstatement of Mc/I, and predicts the permanent curvature (agreement: 0.0827 vs 0.0826 ± 0.0028 m−1).

Known limitations (not fixed). The truth model assumes a perfect bond and an elastic steel strip. It ignores the local contact stresses under the rollers, which affect the bottom gage at x = 150 mm (50 mm from a load roller) in a real test; extrusion residual stresses, which make the onset of yield in a real bar less sharp; and the gages’ transverse sensitivity (about 1 %). Part C consumes a bar.