# Lab 7 — Instructor Key: Deflection of Beams

For instructors only. Do not distribute to students. Every number in this key is computed by src/lab07_analysis.py (and src/make_key.py) from sample_data.csv and src/specimen_measurements.json, or by the simulation’s own model code run in Node. To build a key for your own class data, put your CSV and measurements in the same format and run python3 src/lab07_analysis.py your.csv your_measurements.json, then python3 src/make_key.py. Uncertainties are expanded (U=2u) unless marked u. Sign convention as in the handout (Beer 8e): y up, EI y″=M, M>0 sags, loads down, couples and reaction couples counterclockwise positive; δ=−y is the downward deflection.

## 1. Results at a glance (sample data)

Table K1. Results from the sample data.

| Quantity | Measured | Reference or prediction | Verdict (Eq. 12) |
| --- | --- | --- | --- |
| Bar: b, h (mm) | 25.430 ± 0.026, 6.3620 ± 0.0036 | 25.4 × 6.35 nominal | — |
| E from six Part A compliances (GPa) | 205.01 ± 0.99 | Lab 1: 205.2 ± 2.4 | agree |
| Both loads: δ at 200, 400, 600 mm (mm) | 1.346, 1.927, 1.462 | Eq. 3 from single-load data: 1.345, 1.928, 1.461 | all three agree |
| c13 vs c31 (µm/N) | 41.779 ± 0.161 | 41.714 ± 0.184 | agree |
| B1: EI (N·m²) | 112.31 ± 1.02 | Part A: 111.87 ± 0.54 | agree |
| B1: clamp stiffness k (N·m/rad) | 4 370 ± 310 | — | — |
| B2: RB/P | 0.3308 ± 0.0020 | 5/16 = 0.3125 (Eq. 5); 0.3311 ± 0.0012 (Eq. 8 with k) | disagree with Eq. 5; agree with Eq. 8 |
| B3: ΔRB for 0.500 mm (N) | −0.430 ± 0.021 | −0.489 (Eq. 6); −0.442 ± 0.006 (Eq. 8) | disagree with Eq. 6; agree with Eq. 8 |
| Strip EI, whole-curve fit (N·m²) | 0.1047 ± 0.0008 | linear, four points: 0.1050 ± 0.0012 | consistent |
| Strip E (GPa) | 203.0 ± 3.2 | 203 nominal | — |
| δv at 0.62 kg (mm) | 163.2 ± 0.6 | Eq. 10: 307.6; Eq. 9: 166.3; with weight: 163.5 | agrees only with the weight included |
| α where Eq. 10 is 5 % high | 0.66 (data, interpolated) | 0.676 (Eq. 9) | — |
| D1 lightest section | I 102 × 64 at Ls = 3.05 m | without an overhang: I 152 × 102 | — |

The sample data were generated from a known “truth” model (src/generate_sample_data.py, values in src/truth.json): E = 205 GPa for the bar and 203 GPa for the strip; a clamp of rotational stiffness 4500 N·m/rad in Part B; supports, hangers and indicators a few tenths of a millimeter from their nominal positions; a prop lowered by 0.502 mm instead of 0.500; mass tolerance ±0.05 %; indicator noise 1.5 µm; load-cell noise 0.01 N and a +0.3 % gain error; grid-reading noise 0.25 mm; and the exact elastica with the strip’s own weight. The closure tests in tests/test_lab07_independent.py confirm that the analysis recovers the truth values within their stated uncertainty.

## 2. Pre-lab solutions

Nominal values: bar 25.4 × 6.35 mm, I = 542.0 mm4, E = 205.2 GPa, EI = 111.21 N·m²; strip 12.7 × 0.79 mm, I = 0.5218 mm4, E = 203 GPa, EI = 0.1059 N·m².

P1. (a) P = 3.0 × 9.807 = 29.42 N at a = 0.6 m (b = 0.2 m). Eq. 2 at x = 0.2 and 0.4 m and its mirror image at 0.6 m: δ = 1.235, 1.940, 1.587 mm. Largest moment, under the load: Pab/L = 4.413 N·m; σ=Mc/I = 25.9 MPa, 0.087 of σY. (b) δ=PL3/3EI = 10.08 mm. (c) RB=5P/16 = 12.26 N for P = 39.23 N. (d) α = 6.080 × 0.25²/0.1059 = 3.59; Eq. 10 gives δv = 299 mm, more than the strip’s length of 250 mm: impossible, so Eq. 10 cannot apply at this load. Units: N·m²/(N·m²) for α; N·m³/(N·m²) = m for δ.

P2. Eq. 2 for 2.0 kg at 600 mm gives 1.293 mm at 400 mm; for 1.0 kg at 200 mm, 0.647 mm; Eq. 3: δ400 = 1.940 mm. For the pair 200/600 mm, c12 (indicator 200, load 600) =bxL2−b2−x2/6EIL with b=x = 0.2 m, and c21 (indicator 600, load 200) is the mirror image with a=L−x = 0.2 m: both equal 0.2⋅0.2⋅0.56/6⋅0.8 EI = 41.96 µm/N.

P3. (a) Appendix D.2. (b) RB/P=5/16+0.06/1.12 = 0.3326, above 5/16: a flexible clamp sheds moment to the prop. (c) ΔRB=−3EIΔ/L3 = −0.486 N with L = 0.7 m and Δ = 0.5 mm.

P4. With e=1/EI and f=1/k: cL/2=5L348e+L22f and cL=L33e+L2f. Then cL−2cL/2=L3e/8, so 1/EI=8cL−2cL/2/L3 (the clamp term cancels), and 1/k=16 cL/2−5 cL/3L2. The combination 16cL/2−5cL is zero for a rigid clamp and isolates the clamp. It is a small difference of two large numbers (in the sample data 5.6 % of 16cL/2), so k needs both indicators read to the micrometer; this is why k comes out with a 7 % uncertainty while EI is good to 0.9 %.

P5. Simulation, presets as shipped: “Part A: both loads”: δ = 1.353, 1.942, 1.471 mm at 200, 400 and 600 mm; the parts are 0.824, 1.295, 1.059 mm (2.0 kg at 600 mm) and 0.530, 0.647, 0.412 mm (1.0 kg at 200 mm). “Part B2”: RB = 12.98 N with k = 4400 N·m/rad and 12.26 N with a rigid clamp (5/16 of 39.23 N). “Part C” at 0.62 kg: δv = 163.0 mm and δh = 76.7 mm with the strip’s weight, 162.2 and 75.8 mm without; linear theory is 5 % off at α = 0.676, i.e. at 0.115 kg.

P6. (a) A strip or clamp springing back under load (glasses; clamp and vise tightened before loading; weight lifted off before release; face out of the strip’s plane); falling masses (add one at a time, feet clear); pinch points at the hangers and the prop (indicators set at zero load, no reaching across a loaded hanger). (b) The bar: 3.0 kg on each hanger gives at most 34.5 MPa in Part A, and the Part B limits 40.2 MPa (B1) and 30.2 MPa (B2), all below about one sixth of σY = 296 MPa, so the bar stays linear-elastic with a wide margin and the indicators stay within their travel. The strip: at 0.62 kg the root moment is 1.073 N·m and the bending stress 819 MPa, 0.63 of the 1300 MPa minimum; heavier weights would approach yield at the clamp, where the strip would take a permanent set.

## 3. Analysis solutions

Table K2. Part A compliances (I = 545.7 mm4 from the measured b and h).

| Load at | Indicator at | Compliance c (µm/N) | App. F factor f (×10−3 m³) | E=f/cI (GPa) |
| --- | --- | --- | --- | --- |
| 600 mm | 200 mm | 41.779 ± 0.161 | 4.6667 | 204.7 ± 1.3 |
| 600 mm | 400 mm | 65.546 ± 0.153 | 7.3333 | 205.0 ± 1.1 |
| 600 mm | 600 mm | 53.652 ± 0.176 | 6.0000 | 204.9 ± 1.4 |
| 200 mm | 200 mm | 53.615 ± 0.160 | 6.0000 | 205.1 ± 1.0 |
| 200 mm | 400 mm | 65.458 ± 0.169 | 7.3333 | 205.3 ± 0.9 |
| 200 mm | 600 mm | 41.714 ± 0.184 | 4.6667 | 205.0 ± 1.4 |

A1. Table K2. The six values combine to E = 205.01 ± 0.99 GPa (mean; the spread of the six, 0.61 GPa, is consistent with their uncertainties; the position and I terms are common to all six and are kept in full). Lab 1: 205.2 ± 2.4 GPa: Δ = 0.19 ≤ 2.59, agree. Check: f is in m³ and c in m/N, so f/cI has units m³/(m/N · m4) = N/m². Interpret: the relative standard uncertainties of each coefficient are: positions set with the steel rule (span, hanger, indicator) 0.15–0.29 % through f, the compliance fit 0.12–0.22 %, h 0.08 % (as 3uh/h) and b 0.05 %. The positions limit the accuracy, mainly the span; h is a minor term only because it was measured with a micrometer. With calipers (±0.02 mm), 3uh/h would be 0.54 % and would dominate. h enters as h3 in I; a tension test needs only A=bh (Lab 1), so the same error in h costs three times less there.

A2. Measured with both loads: 1.346, 1.927, 1.462 mm; superposed from the single-load compliances: 1.345, 1.928, 1.461 mm; differences 0.8, 0.5, 0.6 µm against limits 6.0, 6.0, 6.3 µm (Eq. 12): all three agree. App. F with E from A1 gives 1.344, 1.929, 1.461 mm. Reciprocity: c13 = 41.779 ± 0.161 µm/N and c31 = 41.714 ± 0.184 µm/N; Δ = 0.066 ≤ 0.245, agree (0.16 %). The two “middle” compliances, c12 = 65.546 and c32 = 65.458 µm/N, agree by the symmetry of the span, a different property. Check: past yield the moment–curvature relation is no longer linear (Lab 4), so neither superposition nor reciprocity holds: the deflection under both loads would exceed the sum. Interpret: reciprocity holds for any linear-elastic structure, whatever its supports; so an influence line can be measured by moving the indicator and keeping the load fixed (or the reverse), whichever is more convenient.

A3. From the B1 compliances c350 = 374.25 ± 0.90 µm/N and c700 = 1 130.27 ± 1.48 µm/N, P4 gives EI = 112.31 ± 1.02 N·m² and k = 4 370 ± 310 N·m/rad. Part A gave 111.87 ± 0.54 N·m²: Δ = 0.44 ≤ 1.15, agree. From the tip indicator alone with a rigid clamp: EI = 101.16 N·m², 9.6 % low. Check: as k→∞ the second term of Eq. 7 vanishes and δL=PL3/3EI, App. F case 1. Interpret: the clamp contributes 9.9 % of the tip deflection; an unsuspecting student would report E about 185 GPa and blame the material. In this test the clamp is a spring in series with the bar.

A4. Fit of RB against P: RB/P = 0.3308 ± 0.0020. Rigid clamp (Eq. 5): 0.3125, Δ = 0.0183, 9 times U: disagree. Eq. 8 with λ=EI/kL = 0.0367 from A3: 0.3311 ± 0.0012, Δ = 0.0003 ≤ 0.0023, agree. Mid-length deflection per newton: measured 31.45 ± 0.10 µm/N; rigid clamp 27.95 µm/N; with k 31.39 ± 0.34 µm/N (Δ = 0.06 ≤ 0.36, agree with k). Turned around, RB/P alone implies k = 4 450 N·m/rad, consistent with A3. B3: ΔRB = −0.430 ± 0.021 N; Eq. 6 (rigid): −0.489 N (disagree); Eq. 8: −0.442 ± 0.006 N, Δ = 0.012 ≤ 0.022, agree. The indicator at 350 mm moved by 0.168 mm when the prop was lowered. Check: at λ=0, Eq. 8 gives 5/16/1=5/16; as λ→∞, 3λ/2/3λ=1/2, the value for a pin at A. Interpret: the beam is neither fixed nor pinned at A: the clamp is a rotational spring, and with it one parameter measured in B1 predicts two different quantities in B2 and B3. A 0.5 mm settlement changes RB by 4.4 % here; in a simple span (determinate) the reactions follow from statics alone and a settlement only tilts the beam.

Table K3. Part C. EI from the whole-curve fit; Eq. 10 and Eq. 9 columns include the zero-load sag 3.61 mm as an offset. Grid readings ±0.58 mm (U).

| Mass (kg) | α | δv measured (mm) | Eq. 10 (mm) | Eq. 9 (mm) | With weight (mm) | xtip measured (mm) | With weight (mm) | Eq. 10 error (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.000 | 3.2 | 3.6 | 3.6 | 3.6 | 250.4 | 250.4 | — |
| 0.02 | 0.117 | 12.8 | 13.4 | 13.4 | 13.4 | 250.2 | 250.0 | 4.8 |
| 0.04 | 0.235 | 22.6 | 23.2 | 23.1 | 23.0 | 249.0 | 249.2 | 2.8 |
| 0.07 | 0.411 | 37.4 | 37.9 | 37.3 | 37.1 | 246.9 | 247.1 | 1.4 |
| 0.12 | 0.705 | 59.1 | 62.4 | 59.4 | 58.9 | 241.8 | 242.0 | 5.7 |
| 0.17 | 0.998 | 78.4 | 87.0 | 79.1 | 78.2 | 235.2 | 235.3 | 10.9 |
| 0.22 | 1.292 | 94.8 | 111.5 | 96.1 | 94.8 | 227.7 | 227.8 | 17.6 |
| 0.32 | 1.879 | 120.5 | 160.5 | 122.7 | 120.9 | 212.1 | 212.2 | 33.2 |
| 0.42 | 2.467 | 139.6 | 209.5 | 141.9 | 139.6 | 197.7 | 197.5 | 50.1 |
| 0.52 | 3.054 | 153.7 | 258.6 | 155.8 | 153.2 | 184.6 | 184.6 | 68.2 |
| 0.62 | 3.641 | 163.2 | 307.6 | 166.3 | 163.5 | 173.3 | 173.4 | 88.5 |

A5. (a) First four points (0 to 0.07 kg): slope 49.86 mm/N, EI = 0.1050 ± 0.0012 N·m² (L = 250.44 mm from the zero-load reading). (b) Table K3. Eq. 10 is 5 % high at α ≈ 0.66 in the data (Eq. 9: 0.676), and 88 % high at 0.62 kg. Eq. 9 without the weight is within 0.7 mm of the data up to α≈1, then 3.1 mm high at 0.62 kg; the model with the weight matches every point within the grid uncertainty (χ² = 18.1 for 21 degrees of freedom). The tip moves inward by 77.1 mm at 0.62 kg. Check: at α = 0.25, Appendix E gives 0.0827 against α/3 = 0.0833: Eq. 9 approaches Eq. 10. Interpret: as the strip rotates, the load’s lever arm xtip shrinks, so the root moment grows less than in proportion to P and the strip stiffens; at large α part of the load is carried by the strip in tension along its length, not in bending.

A6. Use Table K1. Expected verdicts: E, superposition and reciprocity agree; RB/P and ΔRB disagree with the rigid-clamp formulas and agree with Eq. 8 using the measured k; δv at 0.62 kg disagrees with Eq. 10 by a factor of 1.88, and with Eq. 9 without the weight by 3.1 mm. Full credit needs Table 5 complete, a normalized figure with error bars, and each discrepancy explained with a named, quantified cause (the clamp’s flexibility, large deflection, self-weight).

M1. (a) At 0.62 kg (α = 3.64), Eq. 10 overstates δv by a factor of 1.88; the tip slope is 62.1° (exact model), where dy/dx2 ≈ 3.6 rather than ≪1. (b) Assuming a rigid clamp: RB 5.5 % low (5/16 vs 0.3308 ± 0.0020), ΔRB 14 % too large, EI from the tip 9.6 % low. (c) The weight adds 3.12 mm to δv at 0.12 kg and 0.83 mm at 0.62 kg, against 3.61 mm with no load: as the strip hangs more steeply, the weight’s lever arms shrink. Adding the zero-load sag to the tip-load elastica would be 3.1 mm wrong at 0.62 kg. The data resolve it: the differences exceed the grid U of 0.58 mm (barely at 0.62 kg). (d) With the simulation’s shear term 12fsEI/GAL2 for a central load, shear deformation adds 5 % at a span of 50 mm for the Part A bar (span/depth 7.9) and 1.78 m for the D1 section; at the actual spans it adds 0.02 % (Part A) and 1.7 % (D1 at 3.05 m). Rule: Part A is limited by nothing in the model (the bar is long, stiff and lightly stressed); Part B by the support model; Part C by small slopes, with self-weight second.

## 4. Challenge and Extension solutions

C1. “App. F case 5”: the error is at round-off level (2.2 × 10−9) for every mesh, because the exact solution is piecewise cubic and the mesh has a node under the load; EI y″ is exact too. “App. F case 6”: the nodal deflections are exact for every N; the largest error between nodes falls as N−4.00 and the moment from EI y″ at element midpoints as N−2.00. Reason (Appendix D.6): the Hermite element reproduces any cubic, which is the exact solution of EIy⁗=0, and the consistent load vector makes the nodal values exact; between nodes the cubic misses the quartic part of the uniform-load solution, an error of order h4 in y, and differentiating twice costs h2.

C2. Whole-curve fit with the weight: EI = 0.1047 ± 0.0008 N·m², E = 203.0 ± 3.2 GPa; the four-point linear value 0.1050 ± 0.0012 N·m² is 0.27 % higher, within the uncertainties. Linear theory overstates the deflection already at 0.07 kg (α = 0.41), so the fitted slope is too small and EI too large. The strip’s thickness dominates the uncertainty of E (3ut/t = 0.68 %).

C3. At 4.0 kg, RB = 12.98 N, so 5 % is 0.649 N. With the A3 values, L3/3EI+L2/k = 1.1303 mm/N, and the prop height error that changes RB by 5 % is 0.733 mm, about 370 times the 0.002 mm criterion of step 4. The criterion is conservative for this flexible bar; it matters for stiff beams (Eq. 6 scales with EI/L3), and for the “just touching” condition itself, which defines the zero of the load cell.

X1. Appendix D.5. For small α, expanding the elliptic integrals (or perturbing θ=αθ1+α3θ3 in EIθ″=−Pcosθ) gives θ0=α/2−11α3/240, δv/L=α/3−4α3/105 and δh/L=α2/15 (confirmed numerically by make_key.py: 0.03809 (4/105 = 0.03810), 0.04583 (11/240 = 0.04583) and 0.06667 (1/15 = 0.06667) at α = 0.01). The leading terms are Eq. 10 and the App. F slope PL2/2EI. The inward motion is of second order in α, which is why linear theory has none; the first correction to δv is of third order, so Eq. 10 stays good to 1 % up to α = 0.30.

X2. Appendix D.3. From Eq. 7 extended to a load at a: δx=L=Pa23L−a/6EI+PaL/k for a = 0.35 m, and δx=0.35=Px23L−x/6EI+PLx/k for the tip load: the two expressions are identical term by term, so with the B1 values both give 374.25 µm/N, the measured c350. The clamp term is symmetric in a and x (PaL/k vs PLx/k), so reciprocity survives the spring.

X3. In solveBeam(), where a fixed support locks its rotation, replace the lock by a spring when k is finite: if (std::isfinite(s.k)) K(2*n+1, 2*n+1) += s.k; else lock(2*n+1);, and report the spring moment as -s.k * u[2*n+1]. With the B1 values (EI = 112.31 ± 1.02 N·m², k = 4 370 ± 310 N·m/rad) the extended program gives 11.084 mm at the tip and 3.670 mm at 350 mm under 1.0 kg, the measured 11.084 and 3.670 mm. That agreement is exact by construction (EI and k were found from these two readings); the real test is Part B2, where the same k predicts RB/P = 0.3311 ± 0.0012 against 0.3308 ± 0.0020 measured. The simulation contains exactly this extension (marked [extension] in its source).

## 5. Design problem D1: reference solution

D1. σallow = 0.6 × 273.7 = 164.2 MPa; E = 68.46 GPa (Lab 2). For each section, Ls from 2.50 to 4.00 m in 0.05 m steps; largest M from statics, span deflection from App. F cases 6 and 7, tip from the end slope plus cases 1 and 2 for the overhang (Table K4).

Reference design: I 102 × 64, roller at Ls = 3.05 m (overhang 0.95 m), 3.35 kg/m. Reactions RA = 2 287 N, RB = 7 213 N (check: sum 9 500 N = 2.0 × 4.0 + 1.5 kN). Moment: largest hogging at the roller, MB=−wa2/2+Pa = −2 327.5 N·m; largest sagging 1 307 N·m at x=RA/w = 1.143 m. Stress 55.7 MPa (0.34 of the allowable). Span deflection by superposing case 6 (w on Ls: −15.04 mm at the location of the largest deflection) and case 7 (end couple MB: 8.60 mm): −6.44 mm (limit 8.47 mm). Tip: end slope −0.013 × 10−3 rad × a = −0.01 mm (almost zero: at this roller position the overhang’s moment nearly balances the span’s load), plus case 1 −2.95 mm and case 2 −1.40 mm: −4.36 mm (limit 5.28 mm). The feasible window is narrow: below 3.05 m the tip deflection governs, above 3.10 m the span deflection. The simulation’s preset uses E = 68.9 GPa, so its deflections are 0.6 % smaller; accept either if stated. Without an overhang the lightest section is I 152 × 102 (6.47 kg/m, 1.9 times the mass): the overhang’s hogging moment relieves the span. Assumptions that could invalidate a design: the railing post at the very tip (a post inboard changes the balance); load patterns (live load on the span only, or on the overhang only, can lift the tip or raise the span deflection: a full design checks them); lateral-torsional buckling of the compression flange over the roller, where the bottom flange is in compression and unbraced (Lab 9 territory).

Table K4. D1 candidates. Ratios: σ/σallow, δspan/Ls/360 and δtip/2a/360, at the best roller position for each section (all ratios ≤ 1 there if feasible; otherwise at the position that minimizes the largest ratio).

| Section | I (106 mm4) | Mass (kg/m) | Feasible Ls (m) | Stress ratio | Span deflection ratio | Tip deflection ratio |
| --- | --- | --- | --- | --- | --- | --- |
| I 76 × 51 | 0.809 | 2.29 | none (best 3.05) | 0.67 | 2.00 | 2.17 |
| I 102 × 64 | 2.124 | 3.35 | 3.05–3.10 | 0.34 | 0.76 | 0.83 |
| I 102 × 76 | 2.479 | 3.77 | 3.05–3.15 | 0.29 | 0.65 | 0.71 |
| I 127 × 76 | 4.503 | 4.47 | 2.90–3.40 | 0.20 | 0.36 | 0.39 |
| I 152 × 102 | 9.620 | 6.47 | 2.65–4.00 | 0.11 | 0.17 | 0.18 |
| I 178 × 102 | 14.377 | 7.26 | 2.50–4.00 | 0.09 | 0.11 | 0.12 |
| I 203 × 127 | 25.985 | 9.70 | 2.50–4.00 | 0.06 | 0.06 | 0.07 |

Rubric detail (15 points total in the report). Reactions, moment diagram and section properties (3). Hand superposition matching the simulation within 1 % (3.75). All three checks with the student’s own E and σY and the governing one named (3). At least three sections or roller positions compared quantitatively (3.75). Presentation and assumption (1.5). Typical partial credit: tip deflection not checked (−3); stress taken at the span’s sagging maximum instead of the roller (−2); the overhang’s moment at the roller entered with the wrong sign in case 7 (−2).

## 6. Expected ranges for real class data

Table K5. Expected data ranges.

| Quantity | Expected | If outside, suspect |
| --- | --- | --- |
| E from Part A (GPa) | 200–210 | h by calipers; span measured to the wrong edge of the knife edges; indicator not over the bar’s centerline |
| c13/c31 | 0.99–1.01 | hanger or indicator position error; bar not centered |
| Clamp share of the B1 tip deflection | 5–20 % | loose clamp (> 25 %); clamp edge not where L is measured from |
| RB/P in B2 | 0.32–0.35 | prop not “just touching” at zero load; load cell not tared |
| ΔRB for 0.5 mm (N) | −0.40 to −0.47 | prop screw backlash; reading taken before the load cell settled |
| Strip E (GPa) | 195–210 | thickness by calipers; length from the rule instead of the grid |
| α where Eq. 10 is 5 % high | 0.6–0.8 | parallax in the grid readings; strip twisted in the vise |

## 7. Common student mistakes

- Taking the deflection positive downward without saying so, then writing EI y″=−M in one place and +M in another.

- Using the total mass on the hanger instead of the mass added after zeroing (Part A and B), or forgetting the 0.020 kg hanger in Part C.

- Using the x≤a branch of Eq. 2 for an indicator beyond the load.

- Treating the clamp as rigid in B1, which gives E about 10 % low, and calling it “experimental error”.

- Comparing RB with 5P/16 only, without asking why the gap exceeds U.

- Mixing units in Eq. 6 (Δ in mm with EI in N·m²), giving an answer 1000 times too large.

- Applying α/3 at every Part C load, or measuring the strip’s length with the rule after it has deflected.

- Measuring the strip’s thickness with calipers: ±0.02 mm on 0.79 mm is 7.6 % in E.

- In D1, checking only the span (not the tip, which can deflect upward for long overhangs), or taking the stress at the span’s sagging maximum instead of over the roller.

- Adding the zero-load sag to the large-deflection tip deflection, as if self-weight superposed.

## 8. Grading rubric

Table K6. Grading rubric (100 points plus up to 10 Challenge points). Core analysis A1–A6 totals 45.

| Part | Points | Full credit | Common deductions |
| --- | --- | --- | --- |
| Pre-lab | 15 | P1–P6 correct with units; P1 an estimate made before measuring | P1(d) not compared with L (−2) |
| Data & procedure | 10 | Tables 3–4 complete; checks recorded | h not by micrometer (−3) |
| A1 | 9 | six values of E with U, combined, compared with Lab 1 | no position uncertainty (−2) |
| A2 | 7 | superposition at three points and reciprocity, plot | no plot (−2) |
| A3 | 8 | EI and k with U; rigid-clamp comparison | rigid clamp assumed (−5) |
| A4 | 9 | RB/P and ΔRB against both models | only Eq. 5 (−4) |
| A5 | 7 | EI from the slope; three models plotted against α | no xtip comparison (−2) |
| A6 | 5 | complete table, normalized figure, every discrepancy explained | unexplained discrepancy (−1 each) |
| M1 | 10 | (a)–(d) with numbers and a stated rule | (d) missing (−2) |
| D1 | 15 | see Sec. 5 | see Sec. 5 |
| Communication | 5 | labeled axes, units, x±U format | each unlabeled axis (−1) |
| Challenge | +10 | C1 (3), C2 (3), C3 (4) | — |

Every Core answer must include the required check and interpretation; deduct 1 point per question where either is missing.

## 9. Instructor notes

Before the lab. The bar: 1018 hot-rolled flat (the Lab 1 material, so that Table 1 applies), 25.4 × 6.35 × 1000 mm, straight to 0.5 mm over its length and free of loose scale where the indicators bear (a cold-drawn bar also works, but its yield strength is higher than Table 1); mark 200, 400, 600 mm from one end with a scribe. The frame: two knife edges on a rail, a bench clamp with a hardened jaw edge (the edge defines x = 0 in Part B), and a roller prop on a 50 N load cell with a micrometer screw (or a fine-thread bolt with a dial indicator). Three 0.001 mm digital indicators on magnetic stands. The strip: blue-tempered spring steel shim 12.7 × 0.79 mm (0.031 in), 300 mm long, with a 3 mm hole drilled 250 mm from the vise jaws (drill it before hardening, or use an S-hook over the end); a grid board with 10 mm squares behind it, and a square to sight across.

Timing (2 h). 0:00–0:10 safety and pre-lab; 0:10–0:20 dimensions; 0:20–0:50 Part A; 0:50–1:25 Part B (clamping, B1, prop, B2, B3); 1:25–1:50 Part C; 1:50–2:00 simulation (step 7). If time is short, skip the 200 mm single-load run in Part A (reciprocity then uses the both-loads run and P2), or use the sample data for Part C.

Safety. The strip stores energy: the most likely injury is a strip slipping out of the vise. Inspect the vise jaws, and have students lift the weight before touching the vise. The prop must never carry load while being adjusted beyond the B3 step.

Running without equipment. Give students sample_data.csv and Appendix A; everything in this key applies unchanged. A good hybrid is to run Parts A and B live and use the sample data for Part C.

Your own class data. Export the columns listed in Table A1; fill a measurements JSON like src/specimen_measurements.json, with the class’s Lab 1 and Lab 2 values in reference; run src/lab07_analysis.py and src/make_key.py.

The simulation and the C++ program. Students use the simulation in P5, step 7, A5, M1, C1 and D1. Its finite-element solver is the instructor’s beam_deflection.cpp (in reference/), ported line by line and extended with a clamp spring and a settlement; tests/test_lab07_model.js compiles the C++ and compares the two at every node for ten beams (bit-identical). The readout separates verification rows (“agree”) from model comparisons (“different models”); all tests are summarized in tests/VERIFICATION.md.

## 10. Self-critique and revisions

Before release, the lab was reviewed as a demanding instructor at a top program would review it. The three biggest weaknesses found, and the fixes made:

- A simple-span deflection test only re-confirms App. F, a verification with nothing at stake. Fix: Part B makes the beam indeterminate with a real bench clamp. The textbook value 5P/16 is then measurably wrong (5.5 % low), two indicators in B1 measure the clamp as a rotational spring, and that one measured k predicts two other quantities (RB/P and ΔRB) within their uncertainty: a model refined from data and tested on independent measurements.

- Small-deflection theory was never pushed to failure. Fix: Part C bends a spring-steel strip until Eq. 10 is 88 % high. The exact elastica, tabulated in Appendix E and computed two ways in the simulation, matches the data only when the strip’s weight is included, and M1 shows that the weight does not superpose at large deflection. The measured 5 % point (0.66) is close to the theoretical 0.676.

- The simulation’s “independent” numerical method would be meaningless if the instructor’s C++ program were only paraphrased. Fix: the page ports the C++ solver line by line; the Node tests compile the original and compare the two at every node for ten beams (identical to the last bit); the two extensions (clamp spring, settlement) are tested against the closed forms and against an independent double integration in Python.

Known limitations (not fixed). The indicators push on the bar with about 1 N; zeroing at no load removes it from the increments, but in B2 it slightly preloads the prop. The clamp is modeled as a linear spring; a real clamp can have a small dead zone before it grips, which would show up as a nonzero intercept in B1. Knife-edge friction and the bar’s initial curvature are ignored. The strip is 16 times as wide as it is thick: at the root at 0.62 kg, b2/ρt reaches about 2.1, where the anticlastic curvature starts to be restrained and the strip stiffens toward the plate value EI/1−ν2, up to 9 %; the truth model ignores it, and real data may show E a few percent high from the largest loads.